# Wireless Architecture Options & PCB Redesign

12 BLE sensor nodes + 1 master | Live-session transport | Production architecture review

Decision objective Select a master architecture that can reliably maintain 12 simultaneous node links, aggregate live sensor data with adequate latency margin, and deliver the live stream to a mobile device without forcing an unnecessary rewrite of the existing STM32 node firmware.

This document records engineering options and expected behavior. Final selection is conditional on prototype measurements under full-suit RF conditions.

# Executive Summary

The current single-STM32WB55 master architecture is not suitable for a 12-node production suit because the required connection count exceeds the practical link configuration of the present platform. The redesigned system should preserve STM32WB55 on the sensor nodes, preserve local recording, and change the master architecture only.

Two architecture families remain credible. The lowest-migration path is an STM32WBA65-based master, because STM32WBA can be configured for up to 20 simultaneous BLE connections. The highest-integration/high-bandwidth alternative is an ESP32-C5 master, retaining STM32WB55 nodes while using BLE for the body network and 5 GHz Wi-Fi for the phone-facing link. A maximum-headroom all-STM design using STM32WBA65 + STM32N657 + a 5 GHz Wi-Fi coprocessor is technically strong but substantially more complex.

Current shortlist Prototype and compare: (A) STM32WBA65 with a dedicated second phone-radio path, and (B) ESP32-C5 as the master with STM32WB55 nodes and 5 GHz Wi-Fi to the phone. Keep the STM32N657 architecture as a future/AI-heavy option rather than the first production prototype.

## Key conclusions

Do not change the node MCU solely to solve the master connection problem. The STM32WB55 node already supports the required single BLE link, IMU acquisition and local flash recording.

Do not move the body network to 5 GHz. Bluetooth LE operates in the 2.4 GHz band; 5 GHz would require Wi-Fi or another radio technology and would increase node power/complexity.

The live-session bottleneck is radio scheduling and phone transport, not basic MCU byte-copy processing.

At 4–5 kB/s per node, twelve nodes generate 48–60 kB/s into the master. With the master’s own sensor source, approximately 52–65 kB/s must reach the phone.

If the final model later requires the full current stored payload (~9.6 kB/s per source), the phone-facing requirement can grow to roughly 124.8 kB/s for twelve nodes plus the master; this strongly favors Wi-Fi for the master-to-phone link.

Local-record-first must remain the invariant: BLE/Wi-Fi congestion must never determine whether full-rate sensor data is captured.

## Recommended decision sequence

Freeze the node concept: STM32WB55 + BNO085 + ICM-45686 + local flash; remove RS-485 from the redesigned hardware if BLE-only is confirmed.

Build a WBA65 multi-link proof-of-concept and an ESP32-C5 master proof-of-concept before the production master PCB is frozen.

Run both against the same 12-node traffic generator / real node set and the same mobile-device test set.

Choose based on measured sustained throughput, latency distribution, disconnect rate, buffer growth, RF coexistence behavior, power and firmware effort—not nominal PHY rate.

# Document Map

| Section | Purpose |
| --- | --- |
| 1 | System requirements and traffic model |
| 2 | Baseline: current STM32WB55 master |
| 3 | Option A: dual STM32WB55 master |
| 4 | Option B: single STM32WBA65 master |
| 5 | Option C: STM32WBA65 + dedicated phone radio |
| 6 | Option D: STM32WBA65 + STM32N657 + 5 GHz Wi-Fi |
| 7 | Option E: ESP32-C5 master + STM32WB55 nodes |
| 8 | Nordic alternative (considered, not preferred) |
| 9 | 5 GHz on the body network |
| 10 | Comparison and expected results |
| 11 | Prototype validation plan |
| 12 | PCB and firmware redesign implications |
| 13 | Decision recommendation |
| 14 | References |

# 1. System Requirements and Traffic Model

## 1.1 Target topology

12 x SENSOR NODE PCBs --wireless--> 1 x MASTER PCB --wireless--> MOBILE DEVICE STM32WB55 nodes Master also has local IMUs BNO085 + ICM45686 + flash Live session + recording coordination

The production route requires 12 sensor nodes and one master (13 PCBs active in the suit). The master must simultaneously maintain the node network, acquire its own local sensor data, synchronize/aggregate the streams, and expose the live data to the mobile side.

## 1.2 Current data-rate assumptions

| Case | Per source | 12 nodes → master | Master local | Master → phone |
| --- | --- | --- | --- | --- |
| Observed live – low | 4 kB/s | 48 kB/s | ~4 kB/s | ~52 kB/s |
| Observed live – high | 5 kB/s | 60 kB/s | ~5 kB/s | ~65 kB/s |
| Current full stored payload | 9.6 kB/s | 115.2 kB/s | ~9.6 kB/s | ~124.8 kB/s |

The ~9.6 kB/s stored-payload figure comes from the current sample structures/rates: BNO085 approximately 56 bytes × 100 Hz plus ICM-45686 approximately 20 bytes × 200 Hz. It is not the current live BLE rate; it is included as an important future-headroom scenario.

## 1.3 Single-radio relay workload

If one BLE radio receives all node data and retransmits essentially the same data to the phone, the radio scheduler must service both directions. At the current live rate, this corresponds to roughly 100–125 kB/s of application payload activity across the same RF controller (48–60 kB/s ingress plus 52–65 kB/s egress), before BLE framing, retransmissions and scheduling overhead.

Design implication A stronger CPU does not remove a single-radio airtime limit. The architecture should either reduce the live representation, separate the node and phone radio workloads, or move the phone-facing link to a much higher-throughput technology such as Wi-Fi.

## 1.4 Non-negotiable architectural principle: local-record-first

Each node should continue recording deterministically to local flash at the required sensor rates. Live transport must be a parallel path, not the timing source for acquisition. This keeps sensor capture reliable when BLE/Wi-Fi is congested, temporarily disconnected, or deliberately throttled.

SENSOR ACQUISITION ---> LOCAL FLASH (authoritative full-rate recording) ¦ +-----------> LIVE PACKER / DECIMATOR ---> WIRELESS TRANSPORT

# 2. Baseline: Current STM32WB55 Master

The existing master firmware is already a BLE central to leaf nodes and a BLE peripheral to the operator/client. The current code contains multi-link scheduling, MTU exchange and LE 2M preference, but the WB55 Bluetooth stack is not the appropriate platform for a 12-node + phone production topology.

| Attribute | Current baseline | Assessment |
| --- | --- | --- |
| Node count | Current production firmware logic was developed around a much smaller leaf set | Insufficient for 12-node target |
| Live throughput | Observed approximately 4–5 kB/s per node | Useful prototype result; not the BLE PHY limit |
| Multi-link interval | Current firmware uses a shared multi-link interval around 30–50 ms | Conservative scheduling contributes to current throughput |
| Phone transport | BLE on the same wireless platform | Competes for BLE airtime at full-suit scale |
| Verdict | Keep as reference/prototype implementation | Do not freeze current master PCB for 12-node production |

# 3. Option A: Dual STM32WB55 Master (Split the Nodes)

Nodes 1..6 ---> BLE ---> WB55-A ---- \-- UART/SPI --> Main path --> phone Nodes 7..12 ---> BLE ---> WB55-B ----/

This reuses the two wireless MCUs already present conceptually on the master and divides the node links between them. It is the lowest-risk way to extend the current platform without a new wireless family.

## Advantages

Highest reuse of current STM32WB55 BLE code and debugging knowledge.

Node firmware remains unchanged.

Aggregate node radio load is split across two BLE controllers.

Existing 921.6 kbps inter-MCU UART is theoretically sufficient for approximately 40 kB/s from eight 5 kB/s nodes; a redesigned board should nevertheless prefer SPI + DMA for margin.

## Disadvantages / risks

One controller can operate close to its practical connection ceiling once the phone link is included.

Two 2.4 GHz radios on the same PCB create RF coexistence/layout challenges.

The master firmware becomes a distributed two-controller system with routing, synchronization and failure-state logic.

Limited expansion margin for more nodes or future wireless services.

Expected result Technically feasible as a bridge architecture, but not the preferred long-term production design. It solves link count by partitioning rather than by providing substantial future headroom.

# 4. Option B: Single STM32WBA65 Master

12 x STM32WB55 nodes -- BLE 2M --> STM32WBA65 -- BLE --> Phone + local IMUs

STM32WBA65 is the strongest practical integrated-BLE STM32 candidate discussed for the master. ST documents up to 20 simultaneous connections when the appropriate Link Layer library is used, so 12 node links plus a phone link fit inside the supported connection topology.

| Relevant STM32WBA65 capability | Value / implication |
| --- | --- |
| CPU | Arm Cortex-M33 up to 100 MHz |
| Memory | Up to 2 MB Flash and 512 KB SRAM |
| BLE | LE 2M; configurable multi-link stack |
| Connection topology | ST PM0271 documents up to 20 connections with the 20-link Link Layer library |
| RF | Programmable TX power up to +10 dBm; integrated balun |
| Project fit | Large firmware reuse relative to a vendor migration |

## Advantages

Solves the 13-link topology without splitting nodes across two central controllers.

Keeps the project in STM32Cube/ST wireless APIs and preserves more application code.

Good RAM/Flash headroom for buffers, synchronization and master control.

Single-MCU master is straightforward to debug and manufacture.

## Disadvantages / risks

A single radio must receive 48–60 kB/s from twelve nodes and also transmit 52–65 kB/s toward the phone at the present live rate.

Phone BLE throughput is device-dependent and can become the limiting leg.

Connection-count support does not guarantee adequate aggregate throughput or latency under body-worn RF conditions.

Expected result Likely capable of maintaining the connection count and processing the data. The production risk is single-radio airtime and the phone BLE leg, not CPU capacity. Suitable only if full-load measurements show adequate margin.

# 5. Option C: STM32WBA65 + Dedicated Phone Radio

12 x nodes -- BLE 2.4 GHz --> STM32WBA65 -- SPI/DMA --> 2nd STM32 wireless MCU -- BLE --> phone node network phone link only

This keeps the full sensor network on one WBA65 while moving the phone connection to a second wireless MCU. The second MCU is not used to split the nodes; it exists specifically to separate the two heavy RF workloads.

## Advantages

WBA65 owns all 12 node links, simplifying synchronization and node-state management.

Node radio receives only ~48–60 kB/s instead of also retransmitting the phone stream.

Phone radio handles only ~52–65 kB/s outbound plus control traffic.

SPI + DMA makes the inter-MCU hop negligible relative to BLE connection-event latency.

Retains the STM32 ecosystem and a large fraction of the existing application protocol.

## Disadvantages / risks

Still uses two 2.4 GHz radios in close proximity; antenna placement and coexistence testing are critical.

Phone-facing transport is still BLE, so smartphone throughput remains an external dependency.

Requires an inter-MCU transport protocol, queueing, reset/recovery and version compatibility.

Expected result Strongest practical all-STM BLE architecture for the current live-rate requirement. It provides much better radio-scheduler margin than a single WBA65, but it does not remove the BLE throughput ceiling on the phone side.

# 6. Option D: STM32WBA65 + STM32N657 + 5 GHz Wi-Fi

12 x nodes -- BLE --> STM32WBA65 -- high-speed SPI --> STM32N657 -- 5 GHz Wi-Fi module --> phone radio processor main compute / AI

This is the maximum-headroom STM32 architecture discussed. The WBA65 is dedicated to the BLE sensor network while STM32N657 becomes the system processor for buffering, synchronization, DSP, future on-master inference, storage and high-level control. A dual-band/5 GHz Wi-Fi coprocessor provides the phone-facing link.

| STM32N657 capability | Value / implication |
| --- | --- |
| CPU | Arm Cortex-M55 up to 800 MHz |
| SRAM | 4.2 MB |
| AI | Neural-ART accelerator up to 600 GOPS |
| Use in this project | Extremely large headroom for sensor fusion / ML / future processing |
| Important complexity | No integrated BLE; external boot memory / additional system components required |

## Advantages

Maximum STM32 compute headroom and strong future-proofing for edge AI.

Separates BLE node networking from application processing.

5 GHz phone link removes BLE phone throughput as a bottleneck and provides true spectrum separation from the 2.4 GHz body network.

Recorded-session transfer to the phone can be dramatically faster than BLE.

## Disadvantages / risks

Highest BOM, PCB routing, power and firmware complexity of the STM options.

Requires an external 5 GHz Wi-Fi device/module and associated antenna/certification considerations.

STM32N657 is excessive if the master only forwards and lightly processes sensor data.

More complex boot/memory architecture and more components that can fail.

Expected result Best technical headroom, but likely unnecessary for the first production revision unless heavy on-master ML is a firm requirement. Keep as an expansion path or premium future architecture.

# 7. Option E: ESP32-C5 Master + STM32WB55 Nodes

12 x STM32WB55 nodes -- BLE 2.4 GHz --> ESP32-C5 -- 5 GHz Wi-Fi --> phone + local IMUs + buffering / aggregation

This keeps all node hardware and node firmware on STM32, but replaces the master wireless/application processor with ESP32-C5. C5 combines BLE and dual-band Wi-Fi 6 (2.4/5 GHz) with a 240 MHz RISC-V processor and external PSRAM support. The phone-facing 65 kB/s live requirement is trivial for Wi-Fi compared with BLE.

| ESP32-C5 capability | Project implication |
| --- | --- |
| CPU | Single-core RISC-V up to 240 MHz |
| On-chip SRAM | 384 KB; external PSRAM supported |
| Wireless | Dual-band Wi-Fi 6 (2.4/5 GHz) + Bluetooth LE |
| BLE PHY | Includes 2 Mbps BLE RF support |
| Wi-Fi mode | Can be configured for 5 GHz-only operation |
| Coexistence | Espressif documents Wi-Fi/BLE RF resource arbitration/time-division coexistence; full-load behavior must be measured |

## Advantages

One master SoC can conceptually cover the BLE node network and high-bandwidth 5 GHz phone link.

Node firmware remains STM32WB55, preserving the largest existing body of firmware.

Master-to-phone bandwidth headroom becomes extremely large relative to the required ~52–65 kB/s live stream.

5 GHz Wi-Fi avoids transmitting the phone data in the same 2.4 GHz band used by the BLE nodes.

Simpler hardware than WBA65 + N657 + external Wi-Fi, and potentially simpler than two BLE MCUs.

## Disadvantages / risks

The master firmware must be ported from STM32 HAL/ST BLE APIs to ESP-IDF/NimBLE/FreeRTOS integration.

BLE and Wi-Fi share RF resources internally; Espressif uses coexistence arbitration/time slicing, so twelve active BLE links plus Wi-Fi must be proven on real hardware.

A datasheet/configurable multi-link capability is not sufficient evidence that twelve simultaneous 5 kB/s BLE links will be production-stable.

External PSRAM is recommended for generous per-node buffering and future growth.

Expected result Potentially the cleanest high-bandwidth master architecture if the 12-link BLE + 5 GHz Wi-Fi coexistence test passes. It materially improves the phone transport without rewriting the nodes. It carries more master-firmware porting risk than STM32WBA.

# 8. Nordic Alternative (Considered, Not Preferred)

Nordic remains a technically capable alternative, but it does not currently provide a strong enough reason to migrate the project away from STM32 nodes or to replace STM32WBA solely for standard BLE multi-link operation.

| Device | Strength | What it changes | Project decision |
| --- | --- | --- | --- |
| nRF5340 | Dual Cortex-M33, strong BLE ecosystem, 2 Mbps BLE | Similar standard-BLE airtime problem to WBA | No compelling migration benefit |
| nRF54H20 | Up to 320 MHz cores, 1 MB RAM, proprietary 2.4 GHz up to 4 Mbps | Could enable a different proprietary node network | Only reconsider if standard BLE architecture fails |

A switch to Nordic becomes strategically justified only if the project decides to change the node-network protocol itself (for example, exploiting a proprietary higher-rate radio). That would be a network-architecture rewrite, not a minor MCU substitution.

# 9. Should the Sensor Nodes Move to 5 GHz?

Answer: No for the current design. Bluetooth LE operates in the 2.4 GHz ISM band. A 5 GHz body network would require Wi-Fi or another radio technology, increasing power, firmware complexity and RF design burden. The useful role for 5 GHz is the master-to-phone link, where high bandwidth and spectral separation are valuable.

## Why 5 GHz is better used only on the master

The body nodes need low-power, short-packet communication; BLE is well matched to this role.

The master-to-phone leg carries the aggregated data from all sources and therefore benefits most from a high-throughput link.

Separating node BLE (2.4 GHz) from phone Wi-Fi (5 GHz) avoids repeating the large outbound stream over BLE.

At the same antenna gains and distance, free-space path loss increases with frequency; 5 GHz therefore does not provide a propagation advantage around a human body.

# 10. Architecture Comparison and Expected Results

| Architecture | 12-node fit | Phone transport | Firmware migration | Expected headroom | Decision |
| --- | --- | --- | --- | --- | --- |
| Current WB55 master | No | BLE | None | Insufficient | Reject for production |
| 2 × WB55 split | Yes, partitioned | BLE | Low | Moderate | Fallback / bridge |
| 1 × WBA65 | Yes (20-link stack) | BLE | Moderate | Moderate | Prototype |
| WBA65 + phone STM radio | Yes | BLE | Moderate | High | Leading all-STM candidate |
| WBA65 + N657 + 5 GHz Wi-Fi | Yes | 5 GHz Wi-Fi | High | Very high | Future / AI-heavy option |
| ESP32-C5 master | Needs full-load proof | 5 GHz Wi-Fi | Master port only | High if coexistence passes | Leading integrated candidate |
| Nordic BLE | Yes on suitable parts | BLE | High | Similar BLE constraint | Defer |

## 10.1 Expected behavior by data-rate case

| Traffic case | Single WBA65 all-BLE | WBA65 + phone radio | ESP32-C5 + 5 GHz Wi-Fi |
| --- | --- | --- | --- |
| 4 kB/s/node (52 kB/s to phone) | Plausible; needs margin test | Comfortable node/phone radio separation | Strong bandwidth margin; coexistence test required |
| 5 kB/s/node (65 kB/s to phone) | Higher risk on phone BLE / single airtime scheduler | Good candidate; phone BLE still must sustain ~65 kB/s | Strong phone margin; BLE ingress remains primary test |
| 9.6 kB/s/source full-rate live (~124.8 kB/s to phone) | Not a preferred BLE-only target | Phone BLE likely becomes limiting | Wi-Fi becomes strongly preferred |

## 10.2 Qualitative scoring

| Option | Node FW reuse | Phone bandwidth | RF separation | Implementation risk | Long-term headroom |
| --- | --- | --- | --- | --- | --- |
| WBA65 single | Excellent | Medium | Low | Medium | Good |
| WBA65 + phone STM | Excellent | Medium | Medium | Medium | Very good |
| WBA65 + N657 + 5G Wi-Fi | Excellent | Excellent | Excellent | High | Maximum |
| ESP32-C5 master | Excellent on nodes / master port | Excellent | Very good | Medium-high until tested | Very good |

# 11. Prototype Validation Plan Before PCB Freeze

The architecture should be selected from measured system behavior, not nominal link-count or PHY specifications. The same acceptance test must be applied to the STM32 and ESP master candidates.

## 11.1 Minimum test configurations

STM32WBA65 development board running 12 simultaneous node connections; initially test without the phone, then with the phone path enabled.

All-STM split-radio prototype: WBA65 as node central plus a dedicated STM wireless device for the phone link over SPI/DMA.

ESP32-C5 development board with 12 STM32WB55 node links while simultaneously streaming over 5 GHz Wi-Fi.

At least three representative phone models if the final transport includes phone BLE/Wi-Fi directly.

## 11.2 Traffic tests

| Test | Traffic | Pass evidence |
| --- | --- | --- |
| Nominal live | 12 × 5 kB/s + master local stream | No sustained queue growth; stable links; acceptable latency/loss |
| Headroom stress | 12 × 6 kB/s + master; outbound target ≥ 80 kB/s | Demonstrates margin above current requirement |
| Raw/future stress | Progressively approach full 9.6 kB/s/source payload | Establishes future ceiling even if not a launch requirement |
| Body RF | Nodes placed at real suit locations on a wearer | No architecture-specific failure hidden by bench placement |
| Interference | Nearby 2.4 GHz Wi-Fi/BLE traffic; 5 GHz phone link where applicable | Stable recovery and bounded latency |
| Long session | At least representative full-session duration | No disconnect accumulation, memory leak or buffer creep |

## 11.3 Metrics to record

Per-node connection interval, negotiated PHY, MTU/DLE and effective application throughput.

Per-node packet sequence gaps, retransmission/recovery statistics and disconnect reasons.

End-to-end sample age/latency: median, 95th percentile, 99th percentile and worst observed.

Master RX queue high-water mark and phone-TX queue high-water mark.

CPU utilization, free heap/RAM, buffer allocation failures and watchdog events.

RF RSSI/link quality by body location, including movement and occlusion.

Master power consumption and thermal behavior during simultaneous BLE + phone transport.

Approval criterion Do not approve an architecture that merely reaches the required rate on a quiet bench. The selected design should show measurable throughput and scheduling margin under full-node, body-worn and interference conditions.

# 12. PCB and Firmware Redesign Implications

## 12.1 Node PCB

Retain STM32WB55 unless another independent node requirement forces a change.

Retain BNO085 + ICM-45686 + local flash architecture.

Remove RS-485 transceiver/protection/connectors if BLE-only is the final production decision.

Re-optimize RF placement/ground keep-out after removing RS-485 circuitry; do not treat the freed area as arbitrary copper/placement space.

Preserve programming/debug access and production test points.

## 12.2 Master PCB – common requirements regardless of final MCU

Antenna placement must be treated as a first-order requirement; the master is the hub for twelve body links.

Provide sufficient RAM/buffering either internally or through PSRAM/external memory.

Use DMA-capable high-speed interfaces between processors/radios when a multi-chip design is selected.

Keep dedicated hardware reset/boot/interrupt lines between host and radio coprocessors.

Provide current measurement points for BLE/Wi-Fi stress testing.

Plan certification/antenna options early if a Wi-Fi module is introduced.

## 12.3 Firmware work that should be reused across all candidates

BLEPipe message format and session/control semantics.

Node recording, flash management and full-rate acquisition.

Sequence numbers, CRC/integrity and transfer recovery concepts.

Per-node ring-buffer abstraction and timestamp normalization.

Live-frame aggregation format separated from raw recording format.

Telemetry counters for RF/queue health so system behavior is measurable in production.

## 12.4 Firmware changes required for 12 nodes even if the MCU changes

Generalize all node masks, arrays, connection slots and session validation from the current smaller topology to at least 12 nodes (preferably 16 for implementation headroom).

Replace single-link assumptions with per-connection state machines and non-blocking GATT procedures.

Use credit-based or scheduled streaming so one noisy/slow node cannot stall all others.

Separate live stream scheduling from bulk recorded-session upload scheduling.

Instrument queue depth, packet age and radio timing from the first prototype build.

# 13. Decision Recommendation

The current evidence does not justify changing the node platform. STM32WB55 should remain on the nodes. The master is the component that must change.

## 13.1 Leading candidate A – lowest ecosystem migration

STM32WBA65 + dedicated phone radio Preferred when minimizing firmware-platform change is the highest priority. It offers an explicit 20-link-capable STM BLE controller for the twelve nodes and separates the phone BLE workload onto another radio. Main residual risk: phone BLE throughput and two-radio 2.4 GHz coexistence.

## 13.2 Leading candidate B – highest integration / phone bandwidth

ESP32-C5 master + STM32WB55 nodes Preferred if the full-load 12-link BLE + 5 GHz Wi-Fi prototype passes. It keeps all nodes and their firmware on STM32 while giving the master a high-bandwidth phone path in one SoC. Main residual risk: ESP BLE/Wi-Fi coexistence and the master firmware port.

## 13.3 Do not choose yet

Do not select between these two solely from specifications. Both require a representative full-load test. The result should be documented as a measured margin: maximum sustainable per-node live rate, latency distribution and error/disconnect behavior.

## 13.4 When to escalate to STM32N657

Select the N657-class architecture only when on-master AI/sensor-fusion requirements are concrete enough to justify its processing capability and the additional PCB/firmware complexity. It is not necessary merely to relay the current 4–5 kB/s/node live stream.

## 13.5 Decision gate

| Observation from prototype | Decision | Reason |
| --- | --- | --- |
| WBA65 + phone radio passes with comfortable margin | Stay all-STM | Lowest ecosystem migration with sufficient performance |
| ESP32-C5 passes and materially outperforms all-STM phone transport | Use ESP32-C5 master | Simpler high-bandwidth master; nodes remain STM |
| Both pass similarly | Prefer all-STM unless Wi-Fi product features are valuable | Reduces master firmware port risk |
| BLE node network itself cannot meet the required full-rate live stream | Revisit live data format or radio/network architecture | Vendor change alone will not remove BLE airtime physics |
| On-master ML requires substantially more compute | Evaluate STM32N657 host architecture | Compute requirement, not link count, becomes the driver |

# 14. References and Evidence Base

Specifications below were checked against current vendor documentation during this architecture review. Product availability and SDK behavior should be rechecked at schematic freeze.

1. STMicroelectronics – STM32WBA65RI product page. Source — 100 MHz Cortex-M33, up to 2 MB Flash / 512 KB SRAM, LE 2M, +10 dBm.

2. STMicroelectronics – PM0271, Bluetooth LE stack programming guidelines (Rev. 19, Mar 2026). Source — STM32WBA multiple-connection timing and up to 20 connections with the appropriate Link Layer library.

3. STMicroelectronics – STM32WBA BLE Stack Integration. Source — WBA BLE stack/library variants and RAM footprints for 8/20-connection variants.

4. STMicroelectronics – STM32N657X0 product page. Source — 800 MHz Cortex-M55, 4.2 MB SRAM, Neural-ART accelerator up to 600 GOPS.

5. Espressif – ESP32-C5 product overview. Source — 240 MHz RISC-V, 384 KB on-chip SRAM, external PSRAM support, dual-band 2.4/5 GHz Wi-Fi 6 + BLE.

6. Espressif – ESP32-C5 WROOM datasheet. Source — Bluetooth LE RF characteristics including 2 Mbps; module RF details.

7. Espressif – ESP32-C5 RF coexistence guide. Source — Wi-Fi/BLE RF-resource coexistence and time-division arbitration behavior.

8. Espressif – ESP32-C5 Wi-Fi driver / band mode. Source — 5 GHz-only Wi-Fi mode and dual-band configuration behavior.

9. Nordic Semiconductor – nRF5340 product page. Source — Dual Cortex-M33 architecture, 512 KB + 64 KB RAM, standard BLE platform comparison.

10. Nordic Semiconductor – nRF54H20 product page. Source — Up to 320 MHz processing, 1 MB RAM and proprietary 2.4 GHz throughput up to 4 Mbps.

11. Vantage Suit project repository. Source — Current STM32WB55 node/master firmware architecture, sample structures/rates, BLE multi-link implementation, local recording and PCB context.

## Engineering assumptions / cautions

Payload-rate figures exclude BLE/Wi-Fi protocol overhead. Smartphone performance is device-dependent and must be tested. ESP32-C5 requires a 12-link coexistence proof; configurable limits are not proof of production performance. Final model live fields/rates may materially change the preferred transport.