Vantare Vantage Suit

Coach Assist Requirements & Implementation Plan

Current rollout scope • implementation sequence • immediate engineering milestone

Project direction: The current release is a coach-controlled motion tracking and analytics system. AI is a future version and is not required for the initial rollout.

Prepared from client requirements and current DATASET_ACQUISITION project status

# 1. Scope in One Page

| Product Layer | Purpose | Status |
| --- | --- | --- |
| Basic / Included | Connect, calibrate, track movement, compare coach target vs actual, manual localized haptics, session history/charts/reports/cloud sync. | Build first |
| Premium / Paid | Advanced analytics, benchmarking, customization, reports, collaboration and automation. | Build after Basic |

Future AI Coach: real-time AI analysis, AI-selected corrections/haptics, adaptive thresholds, AI insights and reports. This is intentionally outside the current rollout.

## Immediate engineering goal

Live suit data → calibration → clean body-motion data → simple 3D arm integration.

This is the smallest useful vertical slice. Once it works, target-vs-actual visualization, coach haptics, charts and reports can be layered on top.

# 2. Client Requirements

The client has separated the current Coach Assist rollout into Basic and Premium, with AI reserved for a later version.

## Basic / Included

Suit connection and calibration

Athlete and session setup

Live movement tracking

Coach-defined movement targets

Coach-controlled localized haptic feedback

Session notes, goals, and progress

Basic session history and comparisons

Basic session charts

Standard report/export and sharing

Basic secure cloud sync and backup

Safety controls and alerts

Basic charts should remain simple: target versus actual movement, movement consistency, haptic corrections, and progress from one session to another.

## Premium / Paid

Advanced movement and performance analytics

Custom training programs and advanced settings

Advanced longitudinal progress analysis

Advanced performance reports

Automatic delivery of advanced performance reports to authorized recipients

Athlete benchmarking against historical performance and approved reference movements

Expanded multi-athlete management

Remote coach review and collaboration

## Future AI Version — not current scope

Full AI Coach Mode

Real-time AI movement analysis

AI prioritization of the most important corrections

Automatic AI-selected localized haptic corrections

Individualized AI baselines and thresholds

Adaptive AI coaching over time

Advanced AI insights and explanations

Advanced AI-generated reports

Automatic delivery of advanced AI performance reports

Advanced AI benchmarking

# 3. What We Need to Build Now

The current release does not need a predictive model to decide whether an exercise is correct. It needs a reliable motion pipeline that lets the coach see and compare movement.

| Suit Nodes | BLE Live Data | Calibration | Motion Engine | 3D/App | Coach Target | Charts / Reports |
| --- | --- | --- | --- | --- | --- | --- |

## The Motion Engine is our next responsibility.

Receive synchronized orientation data from the current N2/N3/N4 live stream.

Calibrate the sensors each time the suit is worn so strap orientation does not directly control the avatar.

Convert sensor orientations into clean body-segment motion values such as upper-arm orientation, forearm orientation and elbow movement.

Output a simple, stable data contract that the 3D/mobile team can map to bones in a rigged model.

Keep raw BLE packet details, sensor drivers and firmware-specific structures hidden from the 3D team.

## Recommended 3D-team data handoff

timestamp_ms upper_arm_orientation: {qx, qy, qz, qw} forearm_orientation: {qx, qy, qz, qw} elbow_relative_rotation_deg upper_arm_deviation_deg health: {n2, n3, n4, synchronized, calibrated}

## Important boundary

The 3D/mechanical team should not need raw accelerometer values, raw gyro registers, BLE packets, STM32 protocol details or model features. Their input should be body-motion data.

# 4. Step-by-Step Implementation Plan

| Phase | Focus | What we do | Deliverable |
| --- | --- | --- | --- |
| 0 | Freeze transport baseline | Use the current DATASET_ACQUISITION live BLE path as the known-good baseline. | Stable N2/N3/N4 live stream for motion work. |
| 1 | Motion Engine prototype | Implement calibration and arm kinematics; start with N4 as upper arm, N2 as forearm and N3 as auxiliary/validation. | Live calibrated orientations, elbow relative rotation, upper-arm deviation and health. |
| 2 | Numeric validation | Test neutral, ~45°, ~90° and ~120° positions; re-strap, recalibrate and repeat; check stability/drift. | Evidence the motion values are repeatable enough for 3D. |
| 3 | 3D integration | Give the clean motion packet to the 3D/app team and map orientations to the corresponding bones. | Physical arm motion reproduced by a simple live 3D arm/avatar. |
| 4 | Coach target + manual haptics | Store a coach-approved target movement, compare target vs actual, and keep haptic control coach-driven. | Target-vs-actual display plus logged coach correction events. |
| 5 | Basic product features | Add athlete/session setup, notes/goals, history, basic charts, reports/export, sharing, cloud sync and safety alerts. | Basic / Included rollout. |
| 6 | Premium features | Add advanced analytics, longitudinal analysis, benchmarking, customization, reports, automation, collaboration and multi-athlete tools. | Premium / Paid rollout. |
| 7 | Future AI Coach | Use accumulated structured motion/session data for autonomous analysis, prioritization, AI-selected haptics and AI reporting. | Future release; not a Coach Assist blocker. |

# 5. Immediate Development Milestone

START HERE: build a minimal Motion Engine diagnostic before building the 3D model, app features, Premium analytics or AI.

## First implementation scope

Use the existing N2/N3/N4 BNO085 quaternion stream at the current live rate.

Add a simple neutral-pose calibration step (hold still for roughly 1–2 seconds).

Produce calibrated upper-arm and forearm orientations.

Calculate an initial elbow relative rotation value and upper-arm deviation value.

Display the values live in a diagnostic panel and log them for testing.

Add automated quaternion/calibration math tests so the motion layer is independently verifiable.

## Initial success checks

| Check | Pass condition |
| --- | --- |
| Calibration | Neutral pose produces a stable zero/reference state and rejects obviously moving calibration data. |
| Elbow tracking | Straight, ~45°, ~90° and ~120° physical positions produce clearly separated and approximately corresponding software values. |
| Remount repeatability | After removing and re-strapping the sensors, recalibration restores similar readings for the same physical positions. |
| Live behavior | Values update continuously without depending on the existing AI model. |
| 3D readiness | A 3D developer can consume the motion packet without knowing the BLE or firmware protocol. |

## What we deliberately do not build in this first milestone

AI/model retraining

A polished full-body avatar

Cloud/backend features

Premium reports and dashboards

Automatic haptic decision-making

New MCU/radio architecture

# 6. Team Responsibility Boundary

| Area | Responsibility |
| --- | --- |
| Embedded / Firmware | Keep suit connection, live streaming, sensor health and localized haptic commands reliable. |
| Motion / Host Software | Calibration, synchronized body-motion state, kinematics, target comparison and analytics. |
| 3D / Mechanical / App Team | Rigged body model, bone mapping, rendering, visual target/actual display and user interaction. |
| Backend / Product | Athlete/session storage, history, cloud sync, reports, sharing, Premium collaboration and automation. |
| Future AI | Consume the structured motion/session data after Coach Assist is stable; do not replace the motion engine. |

## Decision summary

The current rollout is Coach Assist, not AI Coach.

The next engineering work is calibration + arm kinematics, not dataset expansion or model retraining.

The first proof is a live numeric Motion Engine; the 3D avatar follows after those numbers are validated.

The 3D team receives clean segment orientations/joint metrics, not raw sensor/BLE data.

All new session data should be structured so Premium analytics and future AI can reuse it later.

NEXT ACTION Implement the Motion Engine diagnostic on the current DATASET_ACQUISITION baseline and validate live elbow/upper-arm motion before expanding scope.