Vantare Vantage Suit

Coach Assist Requirements, Milestone Alignment
& Movement Tracking Plan

Corrected implementation direction for the current rollout

Purpose: move immediately from planning into a working movement-tracking demonstration while keeping future AI out of the current critical path.

CURRENT PRIORITY Complete Milestone 4 movement tracking: live suit → calibration → generic arm motion → 3D/app data interface.

# 1. Corrected Project Direction

The current rollout is Coach Assist, not AI Coach. AI-supported analysis, automatic correction selection and adaptive AI coaching are future-version features and must not block the current prototype.

| Current rollout | Future rollout |
| --- | --- |
| Coach-controlled movement tracking, targets, haptics, history, charts, reports and analytics. | AI movement interpretation, AI prioritization, automatic AI-selected haptics, adaptive thresholds and AI-generated insights/reports. |

Important correction: we do not calibrate the suit “for a bicep curl.” We calibrate each sensor to the wearer’s body segment. Bicep curl is only the first validation exercise.

# 2. Client Requirements

The client has separated the product into Basic / Included, Premium / Paid, and a Future AI Version. The initial rollout contains only Basic and Premium.

| Basic / Included | Premium / Paid |
| --- | --- |
| Suit connection and calibration | Advanced movement and performance analytics |
| Athlete and session setup | Custom training programs and advanced settings |
| Live movement tracking | Advanced longitudinal progress analysis |
| Coach-defined movement targets | Advanced performance reports and automated delivery |
| Coach-controlled localized haptic feedback | Benchmarking against history and approved reference movements |
| Session notes, goals, progress, history and comparisons | Expanded multi-athlete management |
| Basic charts: target vs actual, consistency, haptics, progress | Remote coach review and collaboration |
| Standard report/export/sharing, cloud sync/backup, safety controls | Automation and other time-saving features |

Future AI (not current scope): Full AI Coach Mode, real-time AI analysis, AI correction prioritization, AI-selected haptics, individualized/adaptive thresholds, AI explanations, AI reports and AI benchmarking.

# 3. Alignment with Existing Milestones

The existing milestones PDF already defines the correct near-term sequence. Our new client clarification tells us how to interpret it.

| Milestone | Relevant contractual deliverables | What it means now |
| --- | --- | --- |
| Milestone 4 — Hardware and App Integration Demo | App pairing, BLE communication, movement tracking, session retrieval, integrated system demo. | This is the immediate target. BLE/live transport is largely working; the missing bridge is calibrated body movement tracking and an app/3D data interface. |
| Milestone 5 — Final Integrated Prototype and Handoff | Integrated demo, haptic feedback, correction logic, final source/docs/handoff. | After movement tracking works, add coach-defined targets, target-vs-actual comparison and coach-controlled localized haptic correction. AI is not required. |

# 4. What “Movement Tracking” Must Mean

Movement tracking must be exercise-independent. The sensors should describe how the body segments are moving. Exercise selection should only determine what target or standard the live movement is compared against.

| Suit / IMUs | BLE / Live Data | Body Calibration | Generic Motion Engine | 3D Avatar | Exercise Target | Charts / Haptics |
| --- | --- | --- | --- | --- | --- | --- |

Example: after one body calibration, a bicep curl and a shoulder press should both move the live avatar appropriately. If “Bicep Curl” is selected while the athlete performs a shoulder press, the avatar should still show the shoulder press; only the target comparison will be inappropriate.

# 5. Initial Body Mapping and Current Limits

| Hardware | Initial role in motion tracking |
| --- | --- |
| N2 | Primary forearm / distal-arm orientation |
| N4 | Primary upper-arm orientation |
| N3 | Auxiliary elbow-area sensor, validation/quality signal and localized haptic position |
| Master IMU | Potential torso reference if the Master is physically mounted on the torso; otherwise torso motion remains a limitation in the first arm-only prototype |

Scope limit: the current N2/N3/N4 configuration can support a useful instrumented arm model, but not honest full-body reconstruction. Lower-body or full-body exercises will require additional segment sensors and/or another global/torso reference.

# 6. Data We Must Provide to the 3D/App Team

The 3D team should not receive raw BLE packets or STM32-specific data. Our side should provide a stable motion packet such as:

timestamp_ms upper_arm_orientation: {qx, qy, qz, qw} forearm_orientation: {qx, qy, qz, qw} elbow_relative_rotation_deg upper_arm_deviation_deg health: {n2, n3, n4, synchronized, calibrated}

Later, this contract can grow to signed elbow flexion, forearm rotation, shoulder/torso-relative motion, ROM, rep phase and other metrics without changing the BLE protocol or 3D-team responsibility.

# 7. Step-by-Step Implementation Plan

| Step | Focus | Deliverable / proof |
| --- | --- | --- |
| 1 | Freeze the current live transport baseline | Use the qualified DATASET_ACQUISITION N2/N3/N4 stream. Do not reopen MCU/radio/AI work unless a measured blocker appears. |
| 2 | Body calibration | Neutral-pose calibration that maps each currently worn PCB orientation to its body segment reference. |
| 3 | Generic Motion Engine | Calibrated upper-arm and forearm orientations plus initial elbow relative rotation and upper-arm movement values at live rate. |
| 4 | Numeric validation | Verify known arm positions, repeat after re-strapping/recalibration, and check short-session stability/drift. |
| 5 | 3D integration interface | Give the clean motion packet to the 3D/app team and demonstrate a simple arm following the wearer. |
| 6 | Exercise target layer | Add bicep curl as the first coach-approved target/reference. Exercise targets do not control the raw avatar pose. |
| 7 | Coach Assist correction loop | Target-vs-actual display + coach-controlled localized haptic feedback + logged correction events. |
| 8 | Basic product features | Session setup, notes/goals, history, basic charts, export/sharing, cloud sync/backup and safety controls. |
| 9 | Premium | Advanced analytics, benchmarking, customization, reports, automation, collaboration and multi-athlete management. |
| 10 | Future AI | Use accumulated structured movement/session data for autonomous analysis and coaching only after Coach Assist is stable. |

# 8. Immediate Development Sprint

START HERE — one working vertical slice: wear suit → press Calibrate → move arm → see live elbow/upper-arm values change correctly.

Add a small motion module in the host/live tool; no firmware change is required for the first pass.

Consume the existing synchronized BNO085 quaternions for N2/N3/N4.

Capture a neutral body pose for approximately 1–2 seconds and store the calibration reference.

Output calibrated N2/N4 segment orientation and an initial elbow relative rotation value.

Show a live diagnostic panel with calibration status, node health, elbow rotation and upper-arm deviation.

Log the motion state so recorded tests can be replayed and compared.

Add unit tests for quaternion normalization, relative rotation and calibration behavior.

# 9. First Acceptance Checks

| Check | Pass condition |
| --- | --- |
| Calibration | A neutral pose creates a stable reference; calibration rejects obvious motion or unhealthy streams. |
| Live movement | Arm movement changes the calculated values continuously without using the existing AI model. |
| Elbow separation | Straight, partial bend and deep bend produce clearly separated elbow-relative values. |
| Remount repeatability | After removing/re-strapping sensors, a new calibration restores reasonably similar results for the same physical pose. |
| Exercise independence | After calibration, different arm motions (e.g., curl vs shoulder-press-like motion) still move the body-segment state; no exercise classifier is needed for visualization. |
| 3D readiness | The app/3D team can consume the output packet without knowing the BLE/firmware protocol. |

# 10. What We Are Deliberately Not Doing Now

Retraining or expanding the current predictive AI model as a rollout dependency.

Automatic AI exercise correction or AI-selected haptics.

Building a polished full-body avatar before the motion numbers are validated.

Redesigning the STM32/BLE architecture without a measured movement-tracking blocker.

Claiming full-body biomechanics from the current three arm nodes.

Building Premium/cloud/reporting features before the core movement path is proven.

# 11. Team Responsibility Boundary

| Area | Responsibility |
| --- | --- |
| Embedded / Firmware | Maintain reliable sensor acquisition, BLE live transport, session retrieval, node health and bounded localized haptic commands. |
| Motion / Host Software | Body calibration, synchronized motion frames, generic arm kinematics, target comparison and structured motion logging. |
| 3D / Mechanical / App Team | Rigged model, bone mapping, rendering, actual/target visualization and application interaction. |
| Backend / Product | Athlete/session storage, cloud sync, history, reports, sharing, Premium collaboration and automation. |
| Future AI | Consume structured movement/session data later; AI must not replace the deterministic motion foundation. |

# 12. Working Decision Summary

Current rollout = Coach Assist; AI = future scope. Immediate contract target = complete Milestone 4 movement tracking and 3D/app integration. Calibration maps sensors to body segments, not to a specific exercise. The Motion Engine is generic and exercise-independent; bicep curl is only the first validation/target exercise. First working proof = Calibrate → move arm → correct live motion values → hand clean orientation/joint data to the 3D team. After that, implement coach-defined targets, target-vs-actual visualization and coach-controlled haptics for Milestone 5.

NEXT ACTION: implement the generic Motion Engine diagnostic on the current DATASET_ACQUISITION live stream and validate it physically before expanding scope.